Lesson 5: Imperial Measurement

Learning Goals:

- To be able to use the imperial system to accurately measure length in feet and inches, to accuracies of ⅛ inch and 116inch.

- To be able to use convert between feet and inches.

Topic: Imperial Measurement

- The Imperial System is mainly used in the United States. Canada uses the metric system and imperial system.

- The imperial system is used in some occupations such as construction.

- In the imperial system there are four main units of measurement that are used -  inches (in)  feet (ft.)  yards (yd.)  miles (mi)

- In the imperial system, inches are divided into half. Each half is then divided in half, etc.

- The smallest fraction of an inch that is generally used is116”.

- There are 12 inches in a foot.

- There are two types of rules with inches. The first type of ruler has each inch split into 16 parts. The second type has each inch split into 8 parts.

Converting from a ruler with 16 parts (116”) to a ruler with 8 parts (⅛”) - Take the denominator and multiply by 2 - Take the numerator and multiply by 2 - Keep the whole number as is if there is one

Converting from a ruler with 8 parts (⅛”) to a ruler with 16 parts(116”) - Take the denominator and divide by 2 - Take the numerator and divide by 2 - Keep the whole number as is if there is one

Converting Among Imperial Units - Converting Feet and Inches to Inches - Multiply the feet by 12 and leave the inches alone - Add the remaining inches to that answer you get above

Converting Among Imperial Units - Converting Inches to Feet and Inches - Figure out how many whole feet you can remove by dividing the total amount of inches by 12 - Calculate any remaining inches by multiplying the whole number you get above by 12 and subtracting that from the total amount of inches

Example(s):

1) Identify the measurement indicated by each arrow

316”

2516”

This is a ruler where each inch is divided into 16 parts.

2) Use a ruler to draw a line of each length to the nearest ⅛”. Write the    measurement below the line

Line up the ruler with the line. Accurately measure where the line ends

The line is 238”.

If you had to convert it to the nearest116” you would multiply the denominator by 2 and the numerator by 2. You would leave the whole number as is.

22x32x8” = 2616”

3) Calculate the length of each diagram as indicated with the arrows. Measurements should be rounded to the nearest ⅛”.

Answer 228”

4) Convert 5 ft. 3 inches to inches

5 ft. 3 inches

Multiply the feet by 12 and leave the inches alone

5 x 12 = 60 inches

Add the remaining inches to that answer you get above

60 inches + 3  inches = 63 inches

5 ft 3 inches is equal to 63 inches

5)  Convert 98” inches to feet

98 inches

Figure out how many whole feet you can remove by dividing by 12

98 ÷ 12  = 8 whole feet

Calculate any remaining inches by multiplying the whole number you get above by 12 and subtracting that from the total amount of inches

8 x 12 = 96 inches

98 inches - 96 inches = 2 inches (remaining)

Therefore 98” is the same as 8’ and 2”

Practice Questions:

1) Identify the measurement indicated by each arrow

2) Use a ruler to draw a line of each length to the nearest ⅛”.

3) Use a ruler to draw a line of each length to the nearest116”. Write the measurement above the line

4) Calculate the length of each diagram as indicated with the line. Measurements should be rounded to the nearest ⅛”.

a)

b)

c)

5) Convert each measurement to inches

- 1 ft 6 inches

- 5 ‘ 4”

- 10 ft 2 inches

- 7 ‘ 6”

- 2 ft 3 inches

- 8 ‘ 10”

6) Convert each measurement to feet

- 109”

- 50 inches

- 97”

- 49 inches

- 66”

Strand 2 Lesson 5 Assessment:

1) Identify the measurement indicated by each arrow

2) Use a ruler to draw a line of each length to the nearest ⅛”. Write the measurement above the line

3) Use a ruler to draw a line of each length to the nearest116”. Write the measurement above the line

4) Calculate the length of each diagram as indicated with the arrows. Measurements should be rounded to the nearest ⅛”.

b)

c)

5) Convert each measurement to inches

- 10 ft. 7 inches

- 8’ 7”

- 3 ft. 2 inches

- 9’ 10”

- 7 ft 10 inches

6) Convert each measurement to feet

- 104”

- 99 inches

- 45 “

- 79 inches

- 207”